Yt : https://youtu.be/o_Oac27AS5w?si=MrGvDa0H6GFK-PmA

Practical 3 : Perform :

∙ Implement SPAN Technologies (Switch Port Analyzer)

∙ Implement SNMP and Syslog

∙ Implement Flexible NetFlow

Implement SPAN Technologies (Switch Port Analyzer)

Address Table

| Device | IP Address |
| --- | --- |
| PC0 | 10.0.0.2 |
| PC1 | 10.0.0.3 |

Commands

monitor session 1 source int f0/0

monitor session 1 destination int f0/2

show monitor session 1

show monitor detail

Implement SNMP

| Device | Interface | IP Address |
| --- | --- | --- |
| PC0 | PC0 | 172.168.1.2 /172.168.1.1 |
| R1 | F0/0 | 172.168.1.1 |
| R2 | F0/0 | 172.168.2.1 |

snmp-server community read ro

snmp-server community write rw

Goto MIB browser and do settings

Implement Syslog

| Device | Interface | IP Address |
| --- | --- | --- |
| PC1 |  | 30.0.0.2/30.0.0.1 |
| R2 | F0/0 server | 10.0.0.1 |
|  | F0/1 PC1 | 30.0.0.1 |
| server |  | 10.0.0.2/10.0.0.1 |

service timestamps log datetime msec

int f0/0.1

check syslog of server

Flow Simulator

int f0/0

ip flow ingress

ip flow egress

ip flow-export destination 10.0.0.2 99

ip flow-export source f0/0

show ip cache flow