https://youtu.be/XK5S9-GpKsw?si=AWQVOUY77vmR5YrY

Packet Tracer - Configure EtherChannel

Objectives

Part 1: Configure Basic Switch Settings

Part 2: Configure an EtherChannel with Cisco PAgP

Part 3: Configure an 802.3ad LACP EtherChannel

Part 4: Configure a Redundant EtherChannel Link

Background

Three switches have just been installed. There are redundant uplinks between the switches. As  configured, only one of these links can be used; otherwise, a bridging loop might occur. However,  using only one link utilizes only half of the available bandwidth. EtherChannel allows up to eight  redundant links to be bundled together into one logical link. In this lab, you will configure Port  Aggregation Protocol (PAgP), a Cisco EtherChannel protocol, and Link Aggregation Control Protocol  (LACP), an IEEE 802.3ad open standard version of EtherChannel.

Before beginning the configuration, review the EtherChannel Configuration Guidelines and  Restrictions listed at the end of this activity.

Port Channel Table

| Channel Group | Ports | Protocol |
| --- | --- | --- |
| 1 | S1 F0/21. F0/22 S3 F0/21, F0/22 | PAgP |
| 2 | S1 G0/1, G0/2 S2 G0/1, G0/2 | LACP |
| 3 | S2 F0/23, F0/24 S3 F0/23, F0/24 | Negotiated LACP |

Instructions

Part 1: Configure Basic Switch Settings

a. Assign each switch a hostname according to the topology diagram.

b. Before beginning the link aggregation between switches, verify the existing configuration of the  ports that connect the switches to ensure that the ports will successfully join the EtherChannels.  Commands that provide information about the state of the switch ports include:

Open configuration windowS1# show interfaces | include Ethernet

S1# show interface status

S1# show interfaces trunk

c. Configure all ports that are required for the EtherChannels as static trunk ports.

Close configuration windowNote: If the ports are configured with DTP dynamic auto mode, and you do not set the mode of  the ports to trunk, the links do not form trunks and remain access ports. The default mode on a  2960 switch is for DTP to be enabled and set to dynamic auto. DTP can be disabled on interfaces  with the switchport nonegotiate command.

Part 2: Configure an EtherChannel with Cisco PAgP

Note: When configuring EtherChannels, it is recommended to shut down the physical ports being  grouped on both devices before configuring them into channel groups. Otherwise, EtherChannel  Misconfig Guard may place these ports into err-disabled state. The ports and port channels can be  re-enabled after EtherChannel is configured.

Step 1: Configure Port Channel 1.

a. The first EtherChannel that is created for this activity aggregates ports F0/21 and F0/22  between S1 and S3. Configure the ports on both switches as static trunk ports.

Open configuration window

b. Use the show interfaces trunk command to ensure that you have an active trunk link for those  two links, and the native VLAN on both links is the same.

S1# show interfaces trunk

Port Mode Encapsulation Status Native vlan

F0/21 on 802.1q trunking 1

F0/22 on 802.1q trunking 1

G0/1 on 802.1q trunking 1

G0/2 on 802.1q trunking 1

<output omitted>

c. On S1 and S3, add ports F0/21 and F0/22 to Port Channel 1 with the channel-group 1 mode  desirable command. The mode desirable option enables the switch to actively negotiate to form  a PAgP link. Note:Interfaces must be shutdown before adding them to the channel group.

S1(config)# interface range f0/21 – 22

S1(config-if-range)# shutdown

S1(config-if-range)# channel-group 1 mode desirable

S1(config-if-range)# no shutdown

S3(config)# interface range f0/21 - 22

S3(config-if-range)# shutdown

S3(config-if-range)# channel-group 1 mode desirable

S3(config-if-range)# no shutdown

The message “Creating a port-channel interface Port-channel 1” should appear on both switches  when the channel-group is configured. This interface designation will appear as Po1 in command  output.

d. Configure the logical interface to become a trunk by first entering the interface port channel number command and then the switchport mode trunk command. Add this  configuration to both switches.

S1(config)# interface port-channel 1

S1(config-if)# switchport mode trunk

S3(config)# interface port-channel 1

S3(config-if)# switchport mode trunk

Step 2: Verify Port Channel 1 status.

a. Issue the show etherchannel summary command on S1 and S3 to verify that EtherChannel is  working on both switches. This command displays the type of EtherChannel, the ports utilized,  and the port states. Command output is shown for S1.

S1# show etherchannel summary

Flags: D - down P - in port-channel

I - stand-alone s - suspended

H - Hot-standby (LACP only)

R - Layer3 S - Layer2

U - in use f - failed to allocate aggregator

u - unsuitable for bundling

w - waiting to be aggregated

d - default port

Number of channel-groups in use: 1

Number of aggregators: 1

Group Port-channel Protocol Ports

------+-------------+-----------+-------------------------------------- --

1 Po1(SU) PAgP F0/21(P) F0/22(P)

b. If the EtherChannel does not come up, shut down the physical interfaces on both ends of the  EtherChannel and then bring them back up again. The show interfaces trunk and show  spanning-tree commands should show the port channel as one logical link.

Close configuration window

Part 3: Configure an 802.3ad LACP EtherChannel

Step 1: Configure Port Channel 2.

a. In 2000, the IEEE released 802.3ad, which is an open standard version of EtherChannel. It is  commonly referred to as LACP. Using the previous commands, configure the link  between S1 and S2, using ports G0/1 and G0/2, as an LACP EtherChannel. You must use a different port channel number on S1 than 1, because you already used that in the previous step.

To configure port channel 2 as LACP, use the interface configuration mode channel group 2 mode active command. Active mode indicates that the switch actively tries to negotiate  that link as LACP, as opposed to PAgP. The configuration of S1 is shown below.

Open configuration windowS1(config)# interface range g0/1 - 2

S1(config-if-range)# shutdown

S1(config-if-range)# channel-group 2 mode active

S1(config-if-range)# no shutdown

S1(config-if-range)# interface port-channel 2

S1(config-if)# switchport mode trunk

Step 2: Verify Port Channel 2 status.

Use the show commands from Part 1 Step 2 to verify the status of Port Channel 2. Look for the  protocol used by each port.

Part 4: Configure a Redundant EtherChannel Link

Step 1: Configure Port Channel 3.

There are various options for the channel-group number mode command:

S2(config)# interface range f0/23 - 24

S2(config-if-range)# channel-group 3 mode ?

active Enable LACP unconditionally

auto Enable PAgP only if a PAgP device is detected

desirable Enable PAgP unconditionally

on Enable Etherchannel only

passive Enable LACP only if a LACP device is detected

a. On switch S2, add ports F0/23 and F0/24 to Port Channel 3 with the channel-group 3 mode  passive command. The passive option indicates that you want the switch to use LACP only if  another LACP device is detected. Statically configure Port Channel 3 as a trunk interface.

S2(config)# interface range f0/23 - 24

S2(config-if-range)# shutdown

S2(config-if-range)# channel-group 3 mode passive

S2(config-if-range)# no shutdown

S2(config-if-range)# interface port-channel 3

S2(config-if)# switchport mode trunk

b. On S3, add ports F0/23 and F0/24 to Port Channel 3 with the channel-group 3 mode  active command. The active option indicates that you want the switch to use LACP  unconditionally. Statically configure Port Channel 3 as a trunk interface.

Step 2: Verify Port Channel 3 status.

a. Use the show commands from Part 1 Step 2 to verify the status of Port Channel 3. Look for the  protocol used by each port.

b. Creating EtherChannel links does not prevent Spanning Tree from detecting switching loops.  View the spanning tree status of the active ports on S1.

S1# show spanning-tree active

VLAN0001

Spanning tree enabled protocol ieee

Root ID Priority 32769

Address 0001.436E.8494

Cost 9

Port 27(Port-channel1)

Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec

Bridge ID Priority 32769 (priority 32768 sys-id-ext 1)

Address 000A.F313.2395

Hello Time 2 sec Max Age 20 sec Forward Delay 15 sec

Aging Time 20

Interface Role Sts Cost Prio.Nbr Type

---------------- ---- --- --------- -------- -------------------------------- Po1 Root FWD 9 128.27 Shr

Po2 Altn BLK 3 128.28 Shr

Port Channel 2 is not operative because Spanning Tree Protocol placed some ports into blocking  mode. Unfortunately, those ports were the Gigabit ports. In this topology, you can restore these  ports by configuring S1to be primary root for VLAN 1. You could also set the priority to 24576.

S1(config)# spanning-tree vlan 1 root primary

or

Close configuration window

S1(config)# spanning-tree vlan 1 priority 24576

You may have to wait for STP to recalculate the tree topology. Press fast-forward if necessary.  Use the show spanning-tree active command to verify that the Gigabit ports are now in the  forwarding state.

EtherChannel Configuration Guidelines and Restrictions

EtherChannel has some specific guidelines that must be followed in order to avoid configuration  problems.

1) All Ethernet interfaces support EtherChannel up to a maximum of eight interfaces with no  requirement that the interfaces be on the same interface module.

2) All interfaces within an EtherChannel must operate at the same speed and duplex.

3) EtherChannel links can function as either single VLAN access ports or as trunk links between  switches.

4) All interfaces in a Layer 2 EtherChannel must be members of the same VLAN or be  configured as trunks.

5) If configured as trunk links, Layer 2 EtherChannel must have the same native VLAN and have  the same VLANs allowed on both switches connected to the trunk.

6) When configuring EtherChannel links, all interfaces should be shutdown prior to beginning the  EtherChannel configuration. When configuration is complete, the links can be re-enabled.

7) After configuring the EtherChannel, verify that all interfaces are in the up/up state.

8) It is possible to configure an EtherChannel as static, or for it to use either PAgP or LACP to  negotiate the EtherChannel connection. The determination of how an EtherChannel is setup  is the value of the channel-group number mode command. Valid values are:

active LACP is enabled unconditionally

passive LACP is enabled only if another LACP-capable device is connected.

desirable PAgP is enabled unconditionally

auto PAgP is enabled only if another PAgP-capable device is connected.

on EtherChannel is enabled, but without either LACP or PAgP.

9) LAN ports can form an EtherChannel using PAgP if the modes are compatible. Compatible  PAgP modes are:

desirable => desirable

desirable => auto

If both interfaces are in auto mode, an Etherchannel cannot form.

10) LAN ports can form an EtherChannel using LACP if the modes are compatible. Compatible  LACP modes are:

active => active

active => passive

If both interfaces are in passive mode, an EtherChannel cannot form using LACP.

11) Channel-group numbers are local to the individual switch. Although this activity uses the  same Channel-group number on either end of the EtherChannel connection, it is not a  requirement. Channel-group 1 (interface po1) on one switch can form an EtherChannel with  Channel-group 5 (interface po5) on another switch.

End of document

Switch S1

Hostname S1

int range f0/21-22,g0/1-2

switchport mode trunk

switchport nonegotiate

Switch S2

Hostname S2

int range f0/21-22,g0/1-2

switchport mode trunk

switchport nonegotiate

Switch S3

Hostname S3

int range f0/21-24

switchport mode trunk

switchport nonegotiate

S1(config)# interface range f0/21 – 22

S1(config-if-range)# shutdown

S1(config-if-range)# channel-group 1 mode desirable S1(config-if-range)# no shutdown

S3(config)# interface range f0/21 - 22

S3(config-if-range)# shutdown

S3(config-if-range)# channel-group 1 mode desirable S3(config-if-range)# no shutdown

S1(config)# interface port-channel 1

S1(config-if)# switchport mode trunk

S3(config)# interface port-channel 1

S3(config-if)# switchport mode trunk

S1# show etherchannel summary

S1(config)# interface range g0/1 - 2

S1(config-if-range)# shutdown

S1(config-if-range)# channel-group 2 mode active S1(config-if-range)# no shutdown

S1(config-if-range)# interface port-channel 2 S1(config-if)# switchport mode trunk

S2(config)# interface range f0/23 - 24

S2(config-if-range)# shutdown

S2(config-if-range)# channel-group 3 mode passive S2(config-if-range)# no shutdown

S2(config-if-range)# interface port-channel 3 S2(config-if)# switchport mode trunk

S1# show spanning-tree active