Lab - Configuring Multi-area OSPFv2

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | Lo0 | 209.165.200.225 | 255.255.255.252 |
|  | Lo1 | 192.168.1.1 | 255.255.255.0 |
|  | Lo2 | 192.168.2.1 | 255.255.255.0 |
|  | S0/0/0 (DCE) | 192.168.12.1 | 255.255.255.252 |
| R2 | Lo6 | 192.168.6.1 | 255.255.255.0 |
|  | S0/0/0 | 192.168.12.2 | 255.255.255.252 |
|  | S0/0/1 (DCE) | 192.168.23.1 | 255.255.255.252 |
| R3 | Lo4 | 192.168.4.1 | 255.255.255.0 |
|  | Lo5 | 192.168.5.1 | 255.255.255.0 |
|  | S0/0/1 | 192.168.23.2 | 255.255.255.252 |

Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure a Multi-area OSPFv2 Network

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 1 of 9

Lab - Configuring Multi-area OSPFv2

Background / Scenario

To make OSPF more efficient and scalable, OSPF supports hierarchical routing using the concept of areas.  An OSPF area is a group of routers that share the same link-state information in their link-state databases  (LSDBs). When a large OSPF area is divided into smaller areas, it is called multi-area OSPF. Multi-area OSPF is useful in larger network deployments to reduce processing and memory overhead.

In the lab, you will configure a multi-area OSPFv2 network.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with  Cisco IOS Release 15.2(4)M3 (universalk9 image). Other routers and Cisco IOS versions can be used.  Depending on the model and Cisco IOS version, the commands available and output produced might vary  from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the  correct interface identifiers.

Note: Make sure that the routers have been erased and have no startup configurations. If you are unsure,  contact your instructor.

Required Resources

∙ 3 Routers (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable) ∙ Console cables to configure the Cisco IOS devices via the console ports

∙ Serial cables as shown in the topology

Part 1: Build the Network and Configure Basic Device Settings In Part 1, you will set up the network topology and configure basic settings on the routers. Step 1: Cable the network as shown in the topology.

Step 2: Initialize and reload the routers as necessary.

Step 3: Configure basic settings for each router.

a. Disable DNS lookup.

b. Configure device name, as shown in the topology.

c. Assign class as the privileged EXEC password.

d. Assign cisco as the console and vty passwords.

e. Configure logging synchronous for the console line.

f. Configure an MOTD banner to warn users that unauthorized access is prohibited.

g. Configure the IP addresses listed in the Addressing Table for all interfaces. DCE interfaces should be  configured with a clock rate of 128000. Bandwidth should be set to 128 Kb/s on all serial interfaces.

h. Copy the running configuration to the startup configuration.

Step 4: Verify Layer 3 connectivity.

Use the show ip interface brief command to verify that the IP addressing is correct and that the interfaces  are active. Verify that each router can ping their neighbor’s serial interface.

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 2 of 9

Lab - Configuring Multi-area OSPFv2

Part 2: Configure a Multi-area OSPFv2 Network

In Part 2, you will configure a multi-area OSPFv2 network with a process ID of 1. All LAN loopback interfaces  should be passive.

Step 1: Identify the OSPF router types in the topology.

Identify the Backbone router(s): ______________________

Identify the Autonomous System Boundary Router(s) (ASBR): __________________  Identify the Area Border Router(s) (ABR): ____________________

Identify the Internal router(s): _______________________

Step 2: Configure OSPF on R1.

a. Configure a router ID of 1.1.1.1 with OSPF process ID of 1.

b. Add the networks for R1 to OSPF.

R1(config-router)# network 192.168.1.0 0.0.0.255 area 1

R1(config-router)# network 192.168.2.0 0.0.0.255 area 1

R1(config-router)# network 192.168.12.0 0.0.0.3 area 0

c. Set LAN loopback interfaces, Lo1 and Lo2, as passive.

d. Create a default route to the Internet using exit interface Lo0.

Note: You may see the “%Default route without gateway, if not a point-to-point interface, may impact  performance” message. This is normal behavior if using a Loopback interface to simulate a default route.

e. Configure OSPF to propagate the routes throughout the OSPF areas.

Step 3: Configure OSPF on R2.

a. Configure a router ID of 2.2.2.2 with OSPF process ID of 1.

b. Add the networks for R2 to OSPF. Add the networks to the correct area. Write the commands used in the  space below.

____________________________________________________________________________________ ____________________________________________________________________________________ ____________________________________________________________________________________ c. Set all LAN loopback interfaces as passive.

Step 4: Configure OSPF on R3.

a. Configure a router ID of 3.3.3.3 with OSPF process ID of 1.

b. Add the networks for R3 to OSPF. Write the commands used in the space below.

____________________________________________________________________________________ ____________________________________________________________________________________ ____________________________________________________________________________________ c. Set all LAN loopback interfaces as passive.

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 3 of 9

Lab - Configuring Multi-area OSPFv2

Step 5: Verify that OSPF settings are correct and adjacencies have been established between  routers.

a. Issue the show ip protocols command to verify OSPF settings on each router. Use this command to  identify the OSPF router types and to determine the networks assigned to each area.

R1# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 1.1.1.1

It is an area border and autonomous system boundary router

Redistributing External Routes from,

Number of areas in this router is 2. 2 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.1.0 0.0.0.255 area 1

192.168.2.0 0.0.0.255 area 1

192.168.12.0 0.0.0.3 area 0

Passive Interface(s):

Loopback1

Loopback2

Routing Information Sources:

Gateway Distance Last Update

2.2.2.2 110 00:01:45

Distance: (default is 110)

R2# show ip protocols

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 2.2.2.2

It is an area border router

Number of areas in this router is 2. 2 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.6.0 0.0.0.255 area 3

192.168.12.0 0.0.0.3 area 0

192.168.23.0 0.0.0.3 area 3

Passive Interface(s):

Loopback6

Routing Information Sources:

Gateway Distance Last Update

3.3.3.3 110 00:01:20

1.1.1.1 110 00:10:12

Distance: (default is 110)

R3# show ip protocols

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 4 of 9

Lab - Configuring Multi-area OSPFv2

*** IP Routing is NSF aware ***

Routing Protocol is "ospf 1"

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Router ID 3.3.3.3

Number of areas in this router is 1. 1 normal 0 stub 0 nssa

Maximum path: 4

Routing for Networks:

192.168.4.0 0.0.0.255 area 3

192.168.5.0 0.0.0.255 area 3

192.168.23.0 0.0.0.3 area 3

Passive Interface(s):

Loopback4

Loopback5

Routing Information Sources:

Gateway Distance Last Update

1.1.1.1 110 00:07:46

2.2.2.2 110 00:07:46

Distance: (default is 110)

What is the OSPF router type for each router?

R1: ______________________________________________________________________________ R2: ______________________________________________________________________________ R3: ______________________________________________________________________________

b. Issue the show ip ospf neighbor command to verify that OSPF adjacencies have been established  between routers.

R1# show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface 2.2.2.2 0 FULL/ - 00:00:34 192.168.12.2 Serial0/0/0

R2# show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface 1.1.1.1 0 FULL/ - 00:00:36 192.168.12.1 Serial0/0/0 3.3.3.3 0 FULL/ - 00:00:36 192.168.23.2 Serial0/0/1

R3# show ip ospf neighbor

Neighbor ID Pri State Dead Time Address Interface 2.2.2.2 0 FULL/ - 00:00:38 192.168.23.1 Serial0/0/1

c. Issue the show ip ospf interface brief command to display a summary of interface route costs. R1# show ip ospf interface brief

Interface PID Area IP Address/Mask Cost State Nbrs F/C Se0/0/0 1 0 192.168.12.1/30 781 P2P 1/1 Lo1 1 1 192.168.1.1/24 1 LOOP 0/0

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 5 of 9

Lab - Configuring Multi-area OSPFv2

Lo2 1 1 192.168.2.1/24 1 LOOP 0/0

R2# show ip ospf interface brief

Interface PID Area IP Address/Mask Cost State Nbrs F/C Se0/0/0 1 0 192.168.12.2/30 781 P2P 1/1

Lo6 1 3 192.168.6.1/24 1 LOOP 0/0

Se0/0/1 1 3 192.168.23.1/30 781 P2P 1/1

R3# show ip ospf interface brief

Interface PID Area IP Address/Mask Cost State Nbrs F/C Lo4 1 3 192.168.4.1/24 1 LOOP 0/0

Lo5 1 3 192.168.5.1/24 1 LOOP 0/0

Se0/0/1 1 3 192.168.23.2/30 781 P2P 1/1

Reflection

What are three advantages for designing a network with multi-area OSPF?

_______________________________________________________________________________________ _______________________________________________________________________________________ _______________________________________________________________________________________

Router Interface Summary Table

| Router Interface Summary |  |  |  |  |
| --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface. |  |  |  |  |

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 6 of 9

Lab - Configuring Multi-area OSPFv2

Router R1

line console 0

logging synchronous

exit

int lo 0

ip address 209.165.200.225 255.255.255.252 exit

int lo 1

ip address 192.168.1.1 255.255.255.0

exit

int lo 2

ip address 192.168.2.1 255.255.255.0

exit

do connections for serial cable with clock rate 128000

Router R2

line console 0

logging synchronous

exit

int lo 6

ip address 192.168.6.1 255.255.255.0

exit

Router R3

line console 0

logging synchronous

exit

int lo 4

ip address 192.168.4.1 255.255.255.252 exit

int lo 5

ip address 192.168.5.1 255.255.255.0

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 7 of 9

Lab - Configuring Multi-area OSPFv2

exit

R1 :

int s0/0/0

bandwidth 128

exit

router ospf 1

router-id 1.1.1.1

do show ip route connected

network 192.168.1.0 0.0.0.255 area 1 network 192.168.2.0 0.0.0.255 area 1 network 192.168.12.0 0.0.0.3 area 0 passive-interface lo 1

passive-interface lo 2

exit

ip route 0.0.0.0 0.0.0.0 lo 0

router ospf 1

default-information originate

Router R2

router ospf 1

router-id 2.2.2.2

do show ip router connected

network 192.168.6.0 0.0.0.255 area 3 network 192.168.12.0 0.0.0.3 area 0 network 192.168.23.0 0.0.0.3 area 3

passive-interface lo 6

Router 3

router ospf 1

router-id 3.3.3.3

do show ip router connected

network 192.168.4.0 0.0.0.255 area 3 network 192.168.5.0 0.0.0.3 area 3 network 192.168.23.0 0.0.0.3 area 3

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 8 of 9

Lab - Configuring Multi-area OSPFv2

passive-interface lo 4 passive-interface lo 5

show ip protocols show ip ospf neighbor

show ip ospf interface

© 2024 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 9 of 9